1. Feature

Power Divider

- Part No: CPD06000M18000A03

Date: July.4th.2018

Revision No: Preliminary

▶ High Performance

▶ High Reliability

▶ RoHS Compliant

2. Electrical Specifications

| Parameter | Specification | Remark |
| --- | --- | --- |
| Frequency Range | 6000-18000MHz |  |
| Insertion loss | ≤1.2dB |  |
| VSWR | ≤1.60 (Input) ¦¦ ≤1.50 (Output) |  |
| Amplitude Balance | ≤±0.6dB |  |
| Phase Balance | ≤±6degree |  |
| Isolation | ≥18dB |  |
| Average Power | 30W ( Forward ) 1W (Reverse) |  |
| Impedance | 50Ω |  |

- Mechanical Specifications

| Parameter | Specifications | Remark |
| --- | --- | --- |
| Dimensions | ## 51.0x38.0x10.0mm |  |
| Connector Type | ## SMA-female |  |
| Surface Finish | ## Grey Painting |  |

- 4. Temperature Specifications

| Parameter | Specifications | Remark |
| --- | --- | --- |
| Operational Temperature | -40 to +70 ℃ |  |
| Storage Temperature | -45 to +85 ℃ |  |

- 5. Outline Drawing  (±0.3mm)

Notes : Input power is rated for load VSWR better than 1.20:1

Specifications are subject to change at any time without any notice.

Total Loss = Insertion Loss + 4.8dB split loss